DXGAT / RampVision — technical audit: the current state

6 October 2026 · Valentyn Fedorov · what the system runs today: the modules and what each checks, the models behind them and how old they are, when each check is active during a turnaround, how long everything takes, and where the same logic is written several times. Based on the architecture review of 7 September and the measurements of 8 September – 1 October.

1 · What runs today and how long it takes

Everything is post-processing. Two cameras at the gate record 1080p at 8 fps in one-minute chunks and upload them; after the turnaround the chunks are merged into one full-turn video per camera; then three jobs run one after another on that file — the General Model (detects everything on the apron), the trackers (follow the aircraft, belt loaders, GSE and people), and one job per check — and the verdicts (Pass / Fail / Not observed) land in MongoDB and on RampVision. A result arrives about 24 hours after the recording: on the median of 148 production videos, 9.5 h for the last chunk to reach the bucket, 2.5 h of merge, 2.4 h of General Model, 1.3 h of trackers and 0.9 h of modules. Each GPU job runs on one Tesla T4 with about 4 CPU cores and 12 GB of memory.

| stage | what it does | models | time on a typical (87-minute) video |
| --- | --- | --- | --- |
| merge | finds, orders and merges the chunks into one video | a small classifier for the capture boundary | 1.7 h waiting + 0.8 h running |
| General Model | detects aircraft, parts, equipment, cones, chocks, people on every frame; decides the camera, the aircraft type and the airline; reads the video twice | 3 YOLO detectors, MobileSAM, an airline detector, two small classifiers | 2.4 h (207 ms per frame — 1.7× slower than the recording) |
| trackers | follows every object over time: identity, stops and movement, who is at which door | 2 re-identification networks, 2 segmentation networks, optical flow | 1.3 h (112 ms per frame) |
| modules (27) | the check logic, one job per module; 13 of them also look at the pixels | 13 of 27 carry models of their own (pose, classifiers, segmentation) | 0.9 h in total; 7 s to 6 min per module on the reference event |
| reporting | writes verdicts, reports and the smart timeline | — | minutes |

2 · The shared models (run once per video for all checks)

| model | what it is for | version | when it runs | size MB | time per frame |
| --- | --- | --- | --- | --- | --- |
| GM YOLOv8m (apron classes) | the main detector: aircraft and parts, cones, chocks, equipment, people | March 2024 | every frame | 6.2 | 8.5 ms |
| Chocks YOLOv8 v4.3 | a second detector just for chocks, at higher resolution | v4.3 | every frame | 22.6 | 3.0 ms |
| Vehicle YOLOv8m | a third detector for vehicles and ground equipment | September 2023 | every frame | 52.0 | 5.8 ms |
| MobileSAM vit_t (main-aircraft mask) | an outline of the main aircraft for its movement state | — | every frame with the main aircraft | 40.7 | small |
| Entity (airline) YOLOv5 | which airline (six logos) | — | every 8th frame, up to 40 hits | 14.7 | small |
| Camera cone/wing EfficientNet-B0 v1.8.1 | which camera this is (cone or wing) | v1.8.1 | every post-arrival frame with the main aircraft, CPU | 16.3 | small |
| Tail classifier | aircraft type (jet / aircraft) | — | aircraft type vote | 0.4 | small |

Trackers: two re-identification networks (one trained for people; for belt loaders and GSE a generic ImageNet ResNet-34 used as is), two segmentation networks (aircraft; belt loader and GSE), classic optical flow on up to 250 points per object, and a noise estimate of the frame every two seconds. Together 26 ms per frame on a modern desktop GPU, 112 ms on the production T4.

3 · The checks: what each module verifies, when it is active, what it looks at

27 module repositories implement the client's 30 checks (the chock module gives three verdicts; “proper belt loader approach” is derived from two others). Window: the part of the turnaround the check is active in. Looks at: the detections it reads from the General Model and the objects it follows through the trackers. Pixels: whether the module also reads the images itself.

| M | module | the check | window · how it decides | looks at (General Model) | follows (trackers) | pixels | own models |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M01 | 3-stop brake check | 3 stop brake check | belt loader at the door · verdict at the event | GSE, pushback, trailer | aircraft, belt loader | no | none |
| M02A | aircraft chocks (3 verdicts) | Nose gear chocks applied immediately; Main gear chocks re… | before departure · verdict at the event | pushback, chocks, cones, trailer, fuel truck, ladder, front whe… | aircraft, people, GSE, belt loader | yes | frame-difference chock classifier (… |
| M03 | cargo doors opened | All cargo bin doors opened and verified | belt loader at the door · Fail if not seen in time | front door, rear door, GSE, pushback, trailer, fuel truck, vehi… | aircraft | no | none |
| M04 | belt loader forward chock | Belt loader forward chock remained in place until unit is… | belt loader at the door · verdict at the event | chocks, cones, rear wheel, engine, rear door, wing, pushback, t… | aircraft, belt loader | no | none |
| M05 | belt loader rear cone | Beltloader rear cone positioned after BL is in place | belt loader at the door · verdict at the event | BL cone, engine cone, wing cone, cones, rear wheel, engine, rea… | aircraft, belt loader | no | none |
| M06 | chocks and cones staged | Chocks and cones available and staged for arrival | before arrival · judges the window before the moment | obstacles, side obstacles, belt loader, GSE, pushback, cones, c… | aircraft | no | none |
| M07 | conditioned air removed | Conditioned air removed 10 mins prior to departure and pr… | before departure · judges the window before the moment | trailer, GSE, belt loader, front wheel, nose, pushback | aircraft | yes | AC hose detector (YOLO11s-seg, 1 cl… |
| M08 | cones removed after GSE clear | Cones removed only after all GSE is clear and chocked | before departure · verdict at the event | engine cone, wing cone, tail cone, GSE, pushback, trailer, belt… | aircraft, belt loader | no | none |
| M09 | cones placed | Cones placed in proper positions and timely | arrival · Fail if not seen in time | wing cone, engine cone, tail cone, tail, GSE, pushback, trailer… | aircraft | no | none |
| M10 | crew present | Crew present 10 minutes prior to aircraft arrival | before arrival · judges the window before the moment | people, GSE, pushback, trailer | aircraft | no | none |
| M11 | FOD walk | FOD walk completed | before arrival · judges the window before the moment | front wheel, GSE, belt loader, pushback, tail, nose | aircraft, people | yes | MoveNet SinglePose Thunder v4 |
| M12 | GSE parked and chocked | Motorized GSE parked and properly chocked | belt loader at the door · verdict at the event | chocks, pushback, trailer, fuel truck, cones, ladder | GSE, people, belt loader | yes | frame-difference chock classifier; RTMPose-l body pose (17 keypoints) |
| M13 | hand signals | All GSE guided into aircraft using approved hand signals | arrival · verdict at the event | front door, rear door, pushback, rear wheel, engine, wing, trai… | aircraft, belt loader, people | yes | HRNet-W48 top-down heatmap pose (CO… |
| M14 | handrails on GSE used | Handrails on GSE being used | belt loader at the door · verdict at the event | people, trailer, GSE, engine, rear wheel, rear door | aircraft, people, belt loader | yes | EfficientNetB0 handrail-extension c…; SegFormer-style handrail segmentati…; DWPose-l whole-body pose (133 keypo…; Hand-contact classifier (blind chec… |
| M15 | lead marshaller and wing walkers | Lead marshaller and wing walkers in correct position | before arrival · judges the window before the moment | tail, wing, belt loader, GSE, pushback, trailer, obstacles | aircraft, people | no | HRNet-W48 top-down heatmap pose (CO…; worker role classifier (SVC); walk-along classifier (RandomForest); walker movement-start models (left,… |
| M16 | pushback pin verification | Pushback operator verifies steering bypass pin installati… | before departure · verdict at the event | pushback, nose, front wheel, belt loader, GSE, trailer, obstacl… | aircraft, people | yes | SimCC ResNet-50 top-down pose (COCO… |
| M17 | post-arrival walk-around | Post-arrival walk around inspection completed accurately;… | arrival · needs the whole event | belt loader, fuel truck, GSE, ladder, trailer, rear wheel, tail… | aircraft, people, belt loader | yes | YOLOv8m-pose (worker keypoints; sin…; Depth Anything V2 metric ViT-B; MobileSAM vit_t (TDV masks); Walk-around canvas CNN (TorchPredic… |
| M18 | safety huddle | Safety huddle conducted at huddle cone | before arrival · judges the window before the moment | cones, belt loader, GSE, pushback, trailer, fuel truck | aircraft, people | yes | none |
| M19 | pre-departure walk-around | Pre-departure walk around completed | before departure · needs the whole event | belt loader, GSE, pushback, trailer, engine, cones, tail, nose | aircraft, people, belt loader | yes | YOLOv8m-pose (worker keypoints, sin…; YOLOv8m-pose rescue re-track (secon…; Depth Anything V2 metric ViT-B; MobileSAM vit_t (TDV masks); Walk-around canvas CNN best_acc.ker… |
| M20 | pushback waits for wing walkers | Pushback does not start until wing walkers are in place a… | pushback / departure · verdict at the event | tail, wing, pushback, front wheel, belt loader, GSE, trailer, o… | aircraft, people | no | none |
| M21 | pushback pathway clear | Pushback pathway confirmed clear of obstacles | pushback / departure · one detection → Fail | pushback, tail, nose, belt loader, GSE, fuel truck, trailer, co… | aircraft | no | none |
| M22 | handrails fully extended | Safety handrails fully extended and used | belt loader at the door · verdict at the event | people, trailer, GSE, rear wheel | aircraft, belt loader | yes | EfficientNetB0 handrail-extension c… |
| M23 | safety vests | Employees wearing safety vests secured to body | whole session · one detection → Fail | vests | people | yes | Swin-T orientation classifier (back…; Swin-T vest classifier (unzipped / …; ResNet-18 vest-presence gate; YOLOv8m-pose (v4 belly-visible gate); ResNet-18 small-band vest CNN (v4_c… |
| M24 | safety zone clear | Safety zone confirmed clear | before arrival · judges the window before the moment | cones, front wheel, people, pushback, belt loader, GSE, fuel tr… | aircraft | yes | MobileSAM vit_t (SamPredictor, box … |
| M26 | steering bypass pin | Steering by-pass pin installed, or steering otherwise byp… | arrival · Fail if not seen in time | people, front wheel, chocks | aircraft, people | yes | HRNet-W48 top-down heatmap pose (CO…; tsai InceptionTime pin-installation…; MinMaxScaler |
| M27 | wing walkers with wands | Wing walkers in proper position and using approved wands | pushback / departure · judges the window before the moment | engine, wing, nose, tail, pushback, belt loader, GSE, trailer, … | aircraft, people | no | HRNet-W48 top-down heatmap pose (CO… |

4 · How long each check runs, and whether it already runs live

Post job: one clean job on the 36-minute reference event. Batch: median cost per recorded frame when the monthly test set runs (the usual way they run today). Live: the module's own work per frame inside the real-time branch, the shared models excluded. Runs live as is: whether the unchanged module gives the same verdict in real time.

| M | module | post job | batch per frame | live per frame | runs live as is |
| --- | --- | --- | --- | --- | --- |
| M23 | safety vests | 370 s | 28.2 ms | 18.8 ms | yes, but only answers when the camera stops |
| M14 | handrails on GSE used | 249 s | 30.7 ms | 12.5 ms | yes, but only answers when the camera stops |
| M27 | wing walkers with wands | 111 s | 3.2 ms | 5.1 ms | yes |
| M13 | hand signals | 91 s | 10.1 ms | 0.3 ms | no — reads the recording twice |
| M24 | safety zone clear | 83 s | 10.3 ms | 0.2 ms | no — reads the recording twice |
| M12 | GSE parked and chocked | 76 s | 6.1 ms | 3.2 ms | yes, but only answers when the camera stops |
| M15 | lead marshaller and wing walkers | 59 s | 5.7 ms | 2.5 ms | yes |
| M22 | handrails fully extended | 46 s | 8.4 ms | 2.4 ms | yes |
| M02A | aircraft chocks (3 verdicts) | 38 s | 5.7 ms | 0.5 ms | no — reads the recording twice |
| M07 | conditioned air removed | 27 s | 2.4 ms | 1.3 ms | yes |
| M16 | pushback pin verification | 27 s | 3.9 ms | 0.4 ms | no — reads the recording twice |
| M26 | steering bypass pin | 26 s | 1.9 ms | 0.7 ms | yes |
| M18 | safety huddle | 22 s | 1.9 ms | 0.9 ms | yes |
| M11 | FOD walk | 21 s | 3.0 ms | 1.3 ms | yes |
| M08 | cones removed after GSE clear | 20 s | 4.9 ms | 1.5 ms | yes |
| M01 | 3-stop brake check | 16 s | 3.4 ms | 0.6 ms | yes |
| M21 | pushback pathway clear | 16 s | 3.4 ms | 0.5 ms | yes (the report shifts slightly) |
| M09 | cones placed | 13 s | 2.3 ms | 0.6 ms | yes (the report shifts slightly) |
| M20 | pushback waits for wing walkers | 13 s | 2.4 ms | 0.3 ms | yes |
| M10 | crew present | 12 s | 1.2 ms | 0.5 ms | yes |
| M04 | belt loader forward chock | 12 s | 2.7 ms | 0.3 ms | yes |
| M05 | belt loader rear cone | 12 s | — | 0.4 ms | yes |
| M03 | cargo doors opened | 8 s | 1.4 ms | 0.9 ms | yes |
| M06 | chocks and cones staged | 7 s | 1.4 ms | 0.1 ms | yes |
| M17 | post-arrival walk-around | own environment | 6.8 ms | — | no — its own Python environment |
| M19 | pre-departure walk-around | own environment | 26.1 ms | — | no — its own Python environment |

Where the time goes. Two modules dominate: safety vests (6 min on the reference event — two transformer classifiers on every person of every frame, the whole session) and handrails on GSE (4 min — four networks per belt-loader visit). The twelve checks that read only detections cost seconds each. In batch the busiest part of a turnaround is the belt loader at the door: nine checks active together, 89 ms of work per recorded frame against a real-time budget of 125 ms. The shared General Model and trackers are about 85 % of a real-time frame; the modules' own work is the small part.

5 · The models inside the modules

Thirteen modules carry models of their own, on top of the shared layer. Size is the weights file.

| M | module | model | kind | version / date | MB |
| --- | --- | --- | --- | --- | --- |
| M02A | aircraft chocks (3 verdicts) | frame-difference chock classifier (rear wheels) | image classifier | — | — |
| M07 | conditioned air removed | AC hose detector (YOLO11s-seg, 1 class) | segmentation | — | — |
| M11 | FOD walk | MoveNet SinglePose Thunder v4 | pose (body keypoints) | — | 25 |
| M12 | GSE parked and chocked | frame-difference chock classifier | image classifier | — | — |
| M12 | GSE parked and chocked | RTMPose-l body pose (17 keypoints) | pose (body keypoints) | — | — |
| M13 | hand signals | HRNet-W48 top-down heatmap pose (COCO-17) | pose (body keypoints) | 2022-09-13 | 269 |
| M14 | handrails on GSE used | EfficientNetB0 handrail-extension classifier | image classifier | 2023-09-26 | — |
| M14 | handrails on GSE used | SegFormer-style handrail segmentation (smp Unet, MiT-… | segmentation | 2023-10-30 | — |
| M14 | handrails on GSE used | DWPose-l whole-body pose (133 keypoints incl. hands) | pose (body keypoints) | — | — |
| M14 | handrails on GSE used | Hand-contact classifier (blind checks) | image classifier | — | — |
| M15 | lead marshaller and wing walkers | HRNet-W48 top-down heatmap pose (COCO-17) | pose (body keypoints) | 2022-09-13 | 269 |
| M15 | lead marshaller and wing walkers | worker role classifier (SVC) | image classifier | — | — |
| M15 | lead marshaller and wing walkers | walk-along classifier (RandomForest) | image classifier | — | 2 |
| M15 | lead marshaller and wing walkers | walker movement-start models (left, right) | model | — | 4 |
| M16 | pushback pin verification | SimCC ResNet-50 top-down pose (COCO-17) | pose (body keypoints) | 2022-09-13 | 178 |
| M17 | post-arrival walk-around | YOLOv8m-pose (worker keypoints; singleton shared with… | pose (body keypoints) | — | — |
| M17 | post-arrival walk-around | Depth Anything V2 metric ViT-B | depth estimation | — | — |
| M17 | post-arrival walk-around | MobileSAM vit_t (TDV masks) | segmentation | — | — |
| M17 | post-arrival walk-around | Walk-around canvas CNN (TorchPredictor CNNModel) | image classifier | — | — |
| M19 | pre-departure walk-around | YOLOv8m-pose (worker keypoints, singleton) | pose (body keypoints) | — | — |
| M19 | pre-departure walk-around | YOLOv8m-pose rescue re-track (second YOLO instance, B… | pose (body keypoints) | — | — |
| M19 | pre-departure walk-around | Depth Anything V2 metric ViT-B | depth estimation | — | — |
| M19 | pre-departure walk-around | MobileSAM vit_t (TDV masks) | segmentation | — | — |
| M19 | pre-departure walk-around | Walk-around canvas CNN best_acc.keras via torch twin | image classifier | — | — |
| M22 | handrails fully extended | EfficientNetB0 handrail-extension classifier | image classifier | 2023-09-26 | — |
| M23 | safety vests | Swin-T orientation classifier (back / head / side) | image classifier | — | 110 |
| M23 | safety vests | Swin-T vest classifier (unzipped / zipped) | image classifier | — | 110 |
| M23 | safety vests | ResNet-18 vest-presence gate | image classifier | — | 45 |
| M23 | safety vests | YOLOv8m-pose (v4 belly-visible gate) | pose (body keypoints) | — | 53 |
| M23 | safety vests | ResNet-18 small-band vest CNN (v4_cnn2) | image classifier | — | 45 |
| M24 | safety zone clear | MobileSAM vit_t (SamPredictor, box prompt) | segmentation | — | — |
| M26 | steering bypass pin | HRNet-W48 top-down heatmap pose (COCO-17) | pose (body keypoints) | 2022-09-13 | 269 |
| M26 | steering bypass pin | tsai InceptionTime pin-installation classifier | image classifier | — | 2 |
| M26 | steering bypass pin | MinMaxScaler | time-series classifier | — | — |
| M27 | wing walkers with wands | HRNet-W48 top-down heatmap pose (COCO-17) | pose (body keypoints) | 2022-09-13 | 269 |

Shared and repeated. One pose network (HRNet-W48, 269 MB) is the same file in four modules — hand signals, steering, lead marshaller, wing walkers — each loading its own copy. EfficientNet-B0 appears six times for different jobs. MobileSAM is in four places. Five different pose stacks serve ten modules. Several weights files sit in the checkouts and are loaded by nobody (a second 255 MB HRNet in all four pose modules, a wand detector, a worker-type classifier, an air-unit detector).

6 · Where the same logic is written several times

23 of the 27 modules work out the stage of the turnaround themselves (when the aircraft arrived, when it left) instead of taking it from one place, and 19 reach into the tracker's internal fields to do it — so every change of the tracker reaches them. The same arrival block is copied word for word in nine modules; six more have their own rules; the code base has five different definitions of “departure”. “Pushback attached” is computed twice, and that is why those two modules read the recording twice. “Belt loader at the door” exists in five variants. Two modules recompute the aircraft type although the General Model publishes it. The modules are pinned to ten different revisions of the shared library, and seven of the checkouts the monthly test runs differ from the revisions the review inspected.

| M | module | what it recomputes on its own |
| --- | --- | --- |
| M01 | 3-stop brake check | belt loader at the door |
| M02A | aircraft chocks (3 verdicts) | arrival, pushback attached |
| M03 | cargo doors opened | arrival, departure |
| M04 | belt loader forward chock | belt loader at the door, aircraft type |
| M05 | belt loader rear cone | belt loader at the door |
| M06 | chocks and cones staged | arrival |
| M09 | cones placed | arrival, departure |
| M10 | crew present | arrival |
| M11 | FOD walk | arrival |
| M12 | GSE parked and chocked | departure, GSE stop / departure |
| M13 | hand signals | arrival, belt loader at the door, aircraft type |
| M14 | handrails on GSE used | belt loader at the door |
| M15 | lead marshaller and wing walkers | arrival, departure |
| M16 | pushback pin verification | pushback attached |
| M17 | post-arrival walk-around | arrival, departure, belt loader at the door, its own tracking |
| M18 | safety huddle | arrival |
| M19 | pre-departure walk-around | arrival, departure, belt loader at the door, its own tracking |
| M20 | pushback waits for wing walkers | arrival, departure, pushback attached |
| M21 | pushback pathway clear | departure |
| M22 | handrails fully extended | belt loader at the door |
| M24 | safety zone clear | arrival, its own tracking |
| M26 | steering bypass pin | arrival |
| M27 | wing walkers with wands | arrival, departure |

7 · What the review flagged: what the code verifies against what the checklist says

| M | module | note |
| --- | --- | --- |
| M01 | 3-stop brake check | Do not replace its recomputed status with generic tracker status without replay comparison; seconds vary with configured FPS. |
| M02A | aircraft chocks (3 verdicts) | Placement is assessed within 30 seconds of aircraft stop. |
| M03 | cargo doors opened | Does not verify a BL target-door relationship or independently distinguish mechanical open/closed state beyond detector-label semantics. |
| M04 | belt loader forward chock | A one-time chock latch does not prove continuous chocking throughout the entire parked interval. |
| M05 | belt loader rear cone | Checklist and code thresholds differ; preserve code behavior as the migration baseline, then approve requirement changes separately. |
| M06 | chocks and cones staged | YAML 6/4 counts are not the active main-code 7/6 counts. |
| M07 | conditioned air removed | Held boxes are counted repeatedly in current history. |
| M08 | cones removed after GSE clear | Despite its name it does NOT test whether GSE is chocked, and its equipment coverage is narrower than all GSE. |
| M09 | cones placed | Obstacle report text says 40% while code tests 65%; inconsistent return arity requires an explicit legacy adapter. |
| M10 | crew present | This measures counts over a window, not continuous presence of the same three named workers for ten minutes. |
| M11 | FOD walk | Not the simple horizontal-crossing rule in the basic description; bbox-derived scale is not calibrated physical distance. |
| M12 | GSE parked and chocked | Different chock evidence channels have different semantics; current truthy fusion must be made explicit before sharing. |
| M13 | hand signals | Raised wrists are a coarse gesture proxy, not an approved signal vocabulary. |
| M14 | handrails on GSE used | Whole-body hands are required; do not substitute body-only pose. |
| M15 | lead marshaller and wing walkers | No separate pose/wand model or lead-marshaller signal proof on the active path. |
| M16 | pushback pin verification | Measures a verification-like worker action, not direct pin visibility. |
| M17 | post-arrival walk-around | Post-arrival rescue/outlier options are disabled by default. |
| M18 | safety huddle | Pinned cv_common ac5098d lacks imported PlaneArrivalDetector. |
| M19 | pre-departure walk-around | Pinned cv_common e4c5e36 lacks imported departure/presence helpers. |
| M20 | pushback waits for wing walkers | Current evidence is post-start co-motion, not a direct proof that everyone was ready before start. |
| M21 | pushback pathway clear | Current hazard class list does not include people. |
| M22 | handrails fully extended | 480 frames is about 60 seconds at 8 FPS, not eight minutes. |
| M23 | safety vests | No keypoint pose despite the model name. |
| M24 | safety zone clear | Upstream tracker does not cover every transport class currently tracked here. |
| M26 | steering bypass pin | Distinct learned installation task from heuristic pin verification. |
| M27 | wing walkers with wands | CRITICAL semantic gap: approved-wand Pass is hard-coded on the active path. |